SPECIFICATIONS & SCOPE OF WORK

Audio-Visual (AV) Integration Specialist — Cape Town Office Installation & VC Integration

1. Requisition & Project Overview

Procurement of an accredited Audio-Visual (AV) Integration Specialist to supply hardware, wall-mount two (2) 65-inch Hisense televisions, perform ceiling cable trunking, and integrate the dual displays with the existing Yealink Meeting Board video conferencing system in the Cape Town office. Scope includes full audio and video calibration and system testing.

| Requisition Type / Category: | IT Equipment hardware and software |
| --- | --- |
| Request Type: | Goods and/or Services |
| Target Location: | HSRC Cape Town Regional Office |
| Service Provider Requirement: | Accredited Audio-Visual (AV) Integration Specialist |

2. Scope of Services

Wall Mounting: Professionally wall-mount two (2) 65-inch Hisense televisions using heavy-duty swivel brackets. The installation must be conducted by an accredited AV Integration Specialist to ensure manufacturer warranties remain valid.

Cabling & Trunking: Route, secure, and enclose display, audio, and power cabling neatly across walls and ceilings using 25mm x 10mm PVC trunking and cable ties.

System Integration: Connect and configure both 65-inch display screens to integrate seamlessly with the primary Yealink Meeting Board video conferencing system in the Cape Town meeting room for dual screen projection.

Testing & Commissioning: Perform complete calibration and testing of video output, display resolution, and audio routing to ensure unified functionality during VC sessions.

3. Hardware & Material Specifications

| Item No. | Item Description & Detailed Specifications | Quantity |
| --- | --- | --- |
| 1 | Swivel TV Brackets: Heavy-duty dual articulating arm full-motion TV wall mount (Fits 32"–80", load capacity up to 50kg, tilt +25°/-25°, swivel +/-60°) | 2 units |
| 2 | HDMI Switch: 3-in-1 Out HDMI Selector Cable / Switch (4K x 2K Ultra HD, plug & play, high-speed stable signal output) | 1 unit |
| 3 | HDMI Cable: 20m Male-to-Male HDMI Cable (4K Ultra HD, Version 1.4, Black) | 1 unit |
| 4 | Power Extensions: 10m Heavy-Duty Power Extension Leads | 2 units |
| 5 | Trunking: 25mm x 10mm PVC Cable Trunking (White) | 20 meters |
| 6 | Accessories: Cable ties and heavy-duty wall-mounting fasteners set | 1 lot |